# Packet Tracer - Configure Router Interfaces

# Addressing Table

| Device | Interface | IP Address/Prefix | Default Gateway |
| --- | --- | --- | --- |
| R1 | G0/0 | 172.16.20.1 /25 | N/A |
| R1 | G0/1 | 172.16.20.129 /25 | N/A |
| R1 | S0/0/0 | 209.165.200.225 /30 | N/A |
| PC1 | NIC | 172.16.20.10 /25 | 172.16.20.1 |
| PC2 | NIC | 172.16.20.138 /25 | 172.16.20.129 |
| R2 | G0/0 | 2001:db8:c0de:12::1/64 | N/A |
| R2 | G0/1 | 2001:db8:c0de:13::1/64 | N/A |
| R2 | S0/0/1 | 2001:db8:c0de:11::1/64 | N/A |
| R2 | S0/0/1 | fe80::2 | N/A |
| PC3 | NIC | 2001:db8:c0de:12::a/64 | fe80::2 |
| PC4 | NIC | 2001:db8:c0de:13::a/64 | fe80::2 |

# Objectives

Part 1: Configure IPv4 Addressing and Verify Connectivity

Part 2: Configure IPv6 Addressing and Verify Connectivity

# Background

Routers R1 and R2 each have two LANs. Your task is to configure the appropriate addressing on each device and verify connectivity between the LANs.

Note: The user EXEC password is cisco. The privileged EXEC password is class.

# Instructions

## Configure IPv4 Addressing and Verify Connectivity

### Assign IPv4 addresses to R1 and LAN devices.

Referring to the Addressing Table, configure IP addressing for R1 LAN interfaces, PC1 and PC2. The serial interface has already configured.

### Verify connectivity.

PC1 and PC2 should be able to ping each other and the Dual Stack Server.

## Configure IPv6 Addressing and Verify Connectivity

### Assign IPv6 addresses to R2 and LAN devices.

Referring to the Addressing Table, configure IP addressing for R2 LAN interfaces, PC3 and PC4. The serial interface is already configured.

### Verify connectivity.

PC3 and PC4 should be able to ping each other and the Dual Stack Server.

End of document