# Packet Tracer - Investigate a VLAN Implementation

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 99 | 172.17.99.31 | 255.255.255.0 | N/A |
| S2 | VLAN 99 | 172.17.99.32 | 255.255.255.0 | N/A |
| S3 | VLAN 99 | 172.17.99.33 | 255.255.255.0 | N/A |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | 172.17.10.1 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | 172.17.20.1 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 | 172.17.30.1 |
| PC4 | NIC | 172.17.10.24 | 255.255.255.0 | 172.17.10.1 |
| PC5 | NIC | 172.17.20.25 | 255.255.255.0 | 172.17.20.1 |
| PC6 | NIC | 172.17.30.26 | 255.255.255.0 | 172.17.30.1 |
| PC7 | NIC | 172.17.10.27 | 255.255.255.0 | 172.17.10.1 |
| PC8 | NIC | 172.17.20.28 | 255.255.255.0 | 172.17.20.1 |
| PC9 | NIC | 172.17.30.29 | 255.255.255.0 | 172.17.30.1 |

# Objectives

Part 1: Observe Broadcast Traffic in a VLAN Implementation

Part 2: Observe Broadcast Traffic without VLANs

# Background

In this activity, you will observe how broadcast traffic is forwarded by the switches when VLANs are configured and when VLANs are not configured.

# Instructions

## Observe Broadcast Traffic in a VLAN Implementation

### Ping from PC1 to PC6.

Wait for all the link lights to turn to green. To accelerate this process, click Fast Forward Time located in the bottom tool bar.

Click the Simulation tab and use the Add Simple PDU tool. Click PC1, and then click PC6.

Click the Capture/Forward button to step through the process. Observe the ARP requests as they traverse the network. When the Buffer Full window appears, click the View Previous Events button.

#### Questions:

Were the pings successful? Explain.

Type your answers here.

Look at the Simulation Panel, where did S3 send the packet after receiving it?

Type your answers here.

In normal operation, when a switch receives a broadcast frame on one of its ports, it forwards the frame out all other ports. Notice that S2 only sends the ARP request out F0/1 to S1. Also notice that S3 only sends the ARP request out F0/11 to PC4. PC1 and PC4 both belong to VLAN 10. PC6 belongs to VLAN 30. Because broadcast traffic is contained within the VLAN, PC6 never receives the ARP request from PC1. Because PC4 is not the destination, it discards the ARP request. The ping from PC1 fails because PC1 never receives an ARP reply.

### Ping from PC1 to PC4.

Click the New button under the Scenario 0 dropdown tab. Now click on the Add Simple PDU icon on the right side of Packet Tracer and ping from PC1 to PC4.

Click the Capture/Forward button to step through the process. Observe the ARP requests as they traverse the network. When the Buffer Full window appears, click the View Previous Events button.

#### Question:

Were the pings successful? Explain.

Type your answers here.

Examine the Simulation Panel.

#### Question:

When the packet reached S1, why does it also forward the packet to PC7?

Type your answers here.

## Observe Broadcast Traffic without VLANs

### Clear the configurations on all three switches and delete the VLAN database.

Return to Realtime mode.

Open configuration window

Delete the startup configuration on all 3 switches.

#### Questions:

What command is used to delete the startup configuration of the switches?

Type your answers here.

Where is the VLAN file stored in the switches?

Type your answers here.

Delete the VLAN file on all 3 switches.

#### Question:

What command deletes the VLAN file stored in the switches?

Type your answers here.

### Reload the switches.

Use the reload command in privileged EXEC mode to reset all the switches. Wait for the entire link to turn green. To accelerate this process, click Fast Forward Time located in the bottom yellow tool bar.

Close configuration window

### Click Capture/Forward to send ARP requests and pings.

After the switches reload and the link lights return to green, the network is ready to forward your ARP and ping traffic.

Select Scenario 0 from the drop-down tab to return to Scenario 0.

From Simulation mode, click the Capture/Forward button to step through the process. Notice that the switches now forward the ARP requests out all ports, except the port on which the ARP request was received. This default action of switches is why VLANs can improve network performance. Broadcast traffic is contained within each VLAN. When the Buffer Full window appears, click the View Previous Events button.

# Reflection Questions

If a PC in VLAN 10 sends a broadcast message, which devices receive it?

Type your answers here.

If a PC in VLAN 20 sends a broadcast message, which devices receive it?

Type your answers here.

If a PC in VLAN 30 sends a broadcast message, which devices receive it?

Type your answers here.

What happens to a frame sent from a PC in VLAN 10 to a PC in VLAN 30?

Type your answers here.

In terms of ports, what are the collision domains on the switch?

Type your answers here.

In terms of ports, what are the broadcast domains on the switch?

Type your answers here.

End of document